| REPORT | TITLE | CONTENTS |
| --- | --- | --- |
| Report 1: Phase I STUDSVIK/N(H)- 04/002 Rev. 0 (09/09/04) Rev. 1 (05/10/04) Rev. 2 (25/04/08) | “Fuel Pellet Isotopic Analyses of Vandellós 2 Rods WZtR165 and WZR0058: Qualification of Method” | Results of the experiments performed to qualify the methods applied in the project: IDA (Isotope Dilution Analysis) using HPLC (High Pressure Liquid Chromatography) combined with HPLC-ICP-MS (Inductively Coupled Plasma Mass Spectrometer) or ICP-MS without separation. This second revision includes a re-analysis of the reference solutions of Ce and Nd, and the corresponding correction of the results. All values nX/238U measured by ICP-MS y HPLC-ICP-MS agree with the values obtained for an adjacent fuel measured in Dimitrovgrad and Harwell. The error in the rods identification has also been corrected (See identifications at the end) |
| Report 2: Phase I STUDSVIK/N(H)-03/026 Rev. 0 (19/05/03) Rev.1 (25/04/08) | “Pellet isotopic analysis of Vandellos II rods WZtR165 and WZR0058 by gamma measurements”. | Results of the isotopic contents determined by γ-scanning of rods WZtR165 y WZR0058. Rod identification error is also corrected. |
| Report 3: Phase I STUDSVIK/N(H)-03/069 Rev. 0 (08/03/04) Rev. 1 (25/04/08) | “Fuel Pellet Isotopic Analyses of Vandellós 2 Rods WZtR165 and WZR0058. Final report” | Results of the first measurement campaign. Rod identification error is corrected. |
| Report 4: Phase I STUDSVIK/N(H)-04/135 Rev. 0 (05/10/04) Rev. 1 (25/04/08) | “Fuel Pellet Isotopic Analyses of Vandellós 2 Rods WZtR165 and WZR0058: Complementary Report” | Results of report 3 showed systematic deviations for some isotopes. The corresponding reference solutions were re-analyzed and the concentrations of Ce, Nd and Eu were identified as erroneous. Rod identification error is corrected. |
| Report 5: Phase II STUDSVIK/N(H)-06/217 Rev. 0 (24/11/06) Rev. 1 (25/04/08) | “Additional Nuclide Analysis on Vandellós Fuel: Re-evaluation of Data from Gamma Scanning of Rods WZR0058 and WZtR165” | γ-scans of rods WZtR165 y WZR0058 were re-analyzed. Rod identification error is corrected. |
| Report 6: Phase II STUDSVIK/N(H)-07/140 Rev. 0 (24/04/08) Rev. 1 (10/02/10) | “Additional Fuel Pellet Isotopic Analyses of Vandellós 2 Rods WZtR160 and WZR0058. Final report” | Results of the second campaign: two solutions of the first campaign were re-analyzed, and two additional fuel samples were measured. Problems detected with Mb, Am and Pu. |
| Datos 1: | Gammascan WMtR113 | Original data and calculations of scan of rod WMtR113, re-analysis and burnup calculations |
| Datos 2: | Isotope Analysis | Original IDA results of the 7 samples of rod WZR0046 y one sample of rod WMtR113: revised values and abstract of the results. |
| Datos 3: | Reevaluation gammascan WZR0046 and WMtR124 | Revision of burnup calculations based on -scan. |
| COM-006998 Rev.1 | Letter of transmission of irradiation data. | Irradiation data of the three fuel rods for high burnup fuel isotope determination |

Error in the rods identification:

WZR 0046 > WZR0058

WMtR113 > WMtR160

WMtR124 > WZtR165